Microscopy Pipeline: Block-by-Block & Line-by-Line Guide

A practical, theoretical, and visual walkthrough of the lysosome detection and cell segmentation workflow.

# 1) Overview

This document explains each major block of the analysis script for 3D microscopy data: loading multi-channel CZI/TIFF, detecting lysosomes with Laplacian of Gaussian (LoG) and distance-transform (DT) radius refinement, segmenting cell bodies in channel 2, mapping detected lysosomes to cells, and exporting quantitative tables and illustrative videos.

Inputs: two-channel 3D stacks (lysosome channel and neuron/channel for cell mask).

Core libraries: NumPy, SciPy, scikit-image, tifffile/czifile, OpenCV, napari, pandas.

Outputs: CSV tables (metrics, counts), MP4/GIF videos (fused views & rotations), PNG screenshots, and an interactive napari session.

# 2) Imports and Environment Setup

import os, re, numpy as np, pandas as pd, cv2, imageio, xml.etree.ElementTree as ET
import czifile, tifffile as tiff
from skimage.feature import blob_log
from skimage.filters import gaussian, threshold_local
from skimage.morphology import remove_small_objects, binary_opening, binary_closing, ball, binary_erosion
from scipy.ndimage import distance_transform_edt as edt, rotate as ndrotate, binary_fill_holes
from skimage.measure import label, regionprops
from skimage.segmentation import watershed, find_boundaries
import napari

*OpenCV* (`cv2`) and *imageio* for frame compositing and video writing.

*czifile* and *tifffile* for reading CZI and TIFF/OME-TIFF and their metadata.

*scikit-image* operators: LoG blobs, Gaussian smoothing, morphology, thresholding, labeling, watershed.

*SciPy ndimage* for distance transform (EDT), rotations, hole filling.

*napari* for interactive visualization and screenshots.

# 3) Quick Theory Snapshots

Fig. 1 – Synthetic blobs used to illustrate LoG-based detection.

Fig. 2 – A smoothly varying background motivates local/adaptive thresholding.

Fig. 3 – Watershed segmentation treats images as topographies.

Fig. 4 – Coordinate axes used for per-cell rotations.

# 4) Helper: refine_radii_via_dt

def refine_radii_via_dt(img3d, blobs, win_px=25, bin_method="sauvola"):
    """
    Refine per-blob radius using a local 2D mask + distance transform (DT) on a single Z slice.
    blobs: [N, 4] with [z, y, x, r_px]. Overwrites r_px using EDT-based radius.
    """
    # pick a local patch around each blob center
    # adaptive binarization (Sauvola/local/Otsu)
    # clean-up (opening, remove small objects)
    # keep the connected component under the center
    # EDT on that component; radius = distance to nearest background pixel
    return out

**Idea:** LoG estimates a scale/radius, but DT provides a more robust local radius by measuring the distance from the center to the nearest background.

**win_px:** Half-size of the XY patch extracted around each (y, x) center for robust local thresholding.

**bin_method:** `sauvola` is generally robust on uneven backgrounds; `local` (mean/gaussian) and `otsu` are alternatives.

After binarization, a light **opening** and **remove_small_objects** suppress speckle noise; EDT then yields a continuous radius in pixels.

Result is written back into `blobs[:, 3]` so subsequent physical-unit conversions use the refined estimate.

# 5) Generic Loader (CZI & TIFF/OME)

def load_any(file_path):
    # Detect extension; branch to CZI or TIFF.
    # For CZI: use czifile to read array and XML; infer 2 channels (C,Z,Y,X) or (Z,C,Y,X).
    # Extract voxel sizes from XML Scaling (X,Y,Z) and convert to µm.
    # For TIFF: use tifffile; prefer OME-XML PhysicalSizeX/Y/Z; else fall back to XResolution/YResolution & spacing tags.
    # Return ch1, ch2, (vx_um, vy_um, vz_um), meta

Handles multiple common axis orders by checking shapes and placing channels appropriately.

Voxel size extraction prioritizes **OME-XML**, then **ImageJ tags**. Units are coerced to µm with safe fallbacks.

If only XY resolution exists, Z-spacing may be derived from `ImageDescription` keys like `spacing` or `SliceSpacing`.

# 6) Physical Units & Voxel Geometry

voxel_um3    = vz_um * vy_um * vx_um
lin_equiv_um = voxel_um3 ** (1.0 / 3.0)

Per-voxel volume (µm³) enables conversion of counts to volumes (e.g., cell volumes from voxel counts).

The cube-root of the voxel volume gives a **linear equivalent** spacing for rough isotropic intuition.

# 7) Lysosome Centers: LoG + Gaussian Pre-filter

image_smooth = gaussian(image, sigma=1.5)
blobs = blob_log(image_smooth,
                 min_sigma=0.8, max_sigma=3.0, num_sigma=5,
                 threshold=0.0035, overlap=1)
if len(blobs) > 0:
    blobs[:, 3] = blobs[:, 3] * np.sqrt(3)
blobs = refine_radii_via_dt(image_smooth, blobs, win_px=25, bin_method="sauvola")

**Gaussian smoothing (σ=1.5)** dampens noise before LoG.

`min_sigma/max_sigma/num_sigma` define the multi-scale search; match to expected lysosome diameters (in pixels).

`threshold` trades sensitivity vs. false positives; lower values find more blobs but risk noise.

`overlap=1` allows full overlap merging logic (here permissive to keep close blobs).

The **√3 scale correction** maps LoG scale to radius (conventional factor for LoG on 3D/2D); refined later by DT.

# 8) Physical Conversion of Blob Metrics

z_um = blobs[:, 0] * vz_um
y_um = blobs[:, 1] * vy_um
x_um = blobs[:, 2] * vx_um
radius_um   = (blobs[:, 3] * vx_um) * 0.1  # NOTE: script applies an extra 0.1 factor
diameter_um = 2.0 * radius_um
volume_um3  = (4/3) * π * radius_um**3

Blob centers (z,y,x) are converted to microns using per-axis voxel sizes.

Radius is from DT (XY pixels) scaled by **vx_um**; the script multiplies by **0.1** as an additional scaling—ensure this matches microscope calibration. Remove or adjust if radii seem 10× too small/large.

# 9) Segmenting Cells from Channel 2

# normalize ch2 to [0,1], then gaussian(sigma=1.5)
# per-slice local threshold (block_size=281, offset=-0.15*std)
# remove_small_objects(..., min_size=20000), closing(ball(10)), erosion(ball(9))
# distance transform → initial cell_mask
# label somas, watershed(-dist_inside, markers=body_lab, mask=neuron_mask)

Local thresholding per Z-slice counteracts axial intensity variations and inhomogeneous illumination.

Morphological **closing** fills gaps; **erosion** tightens borders; **remove_small_objects** removes speckles.

Distance transform inside `neuron_mask` seeds the **watershed**, splitting touching somas into territories.

Tune `block_size`, `offset`, and structuring element radii (`ball(…)`) to match microscope resolution.

# 10) Optional Filtering of Oversized Bodies

MAX_BODY_VOXELS = 65000
# compute voxel counts per label and drop those exceeding threshold

Rejects implausibly large connected components (artifacts/merges).

Adapt threshold to your volume and voxel scaling; inspect histograms of voxel counts.

# 11) Mapping Lysosomes to Cell IDs

for (z,y,x) in blobs[:, :3]:
    if cell_mask[z,y,x]: label = cell_seg[z,y,x] else 0 (outside)
# augment df with location and cell_id; export counts per location and per cell

Each blob is classified as **'cell'** if its center lies inside the binary `cell_mask`; otherwise **'outside'**.

If watershed labels exist, `cell_id_ch2` records the specific cell ID for per-cell statistics.

Exports: `lysosome_counts_cell_vs_outside.csv`, `lysosome_counts_by_cell.csv`, `lysosomes_with_cell_vs_outside.csv`.

# 12) Cell Volumes and Merged Metrics

counts = bincount(cell_seg.ravel())
volume_um3 = counts[1:] * voxel_um3
merge with per-cell lysosome counts → cell_metrics_ch2.csv

Computes per-cell physical volumes (µm³) from label voxel counts and voxel size.

Merges with lysosome counts to facilitate density metrics (e.g., lysosomes per 1000 µm³).

# 13) Napari Overlays & Boundaries

viewer.add_labels(cell_mask, name='Cell (Ch2)')
viewer.add_labels(cell_seg, name='Cell ID (Ch2)')
boundaries = find_boundaries(cell_seg)
viewer.add_image(boundaries, name='Cell ID boundaries', colormap='magenta')

Binary cell mask (green, translucent) provides overall footprint.

Label image displays per-cell territories; boundaries layer highlights watershed edges.

Optional points layer shows only in-cell lysosomes with ID labels for quick QC.

# 14) 2D Slice Videos (Fused & Side-by-Side)

# Fused: overlay cell mask (green) and draw only in-cell lysosomes (yellow/cyan) → ch2_fused_cell.mp4
# Side-by-side: raw Ch1 | segmented Ch2 with overlays → raw_vs_segmented_side_by_side.mp4

Frame-by-frame traversal along Z conveys 3D context for presentations and QC.

Only **in-cell** lysosomes are drawn to emphasize biologically relevant detections.

# 15) Per-Cell Rotation Videos (X, Y, Z)

# For each cell containing lysosomes:
# - extract subvolume and corresponding labels
# - rotate around X/Y/Z (ndrotate), compute projection (MIP), outline cell, plot lysosome points
# - export MP4/GIF per-axis (cell_###_spinX/Y/Z.mp4)

Three axis families provide complementary perspectives on intracellular distribution.

Points are rotated analytically to match rotated projections, ensuring spatial consistency.

Edge-enhanced intensity projections help visualize surface relief without explicit 3D rendering.

# 16) 3D Screenshots via napari

viewer.dims.ndisplay = 3
# camera.angles/elevation/azimuth set for XY/YZ/XZ orientations
viewer.screenshot(canvas_only=True) → PNG

Automates XY/YZ/XZ 3D views for quick documentation.

Useful for manuscripts and lab notebooks to capture the final segmentation context.

# 17) Practical Tuning & Troubleshooting

Voxel scaling: double-check OME-XML or calibration; verify a known ruler object.

LoG scale: align `min_sigma/max_sigma` with expected lysosome sizes in **pixels** (convert from µm).

DT window: `win_px` should be large enough to enclose each lysosome but small enough to avoid neighbors.

Adaptive thresholding: start with `sauvola`, then experiment with `offset` and `block_size` if backgrounds vary.

Morphology radii (`ball`): choose comparable to soma boundary thickness in voxels; excessive operations may erode small neurites.

Watershed markers: consider pre-labeling robust somas as markers to avoid spurious splits or merges.

QC: overlay detected points and IDs in napari; spot-check a few slices for false positives/negatives.

Scaling factor in radius: confirm the extra ×0.1 is desired; remove if mismatched to ground truth spheres.

Performance: for large volumes, consider chunking or restricting LoG scales; avoid overly large `block_size` in thresholding.

# 18) Minimal Parameter Map (What to tweak first)

**LoG**: `min_sigma`, `max_sigma`, `threshold`

**DT refinement**: `win_px`, `bin_method`

**Thresholding (Ch2)**: `block_size`, `offset`

**Morphology**: radii in `ball()`, `min_size` in `remove_small_objects`

**Watershed**: marker quality (derived from distance peaks) and mask quality

# 19) Closing Notes

The pipeline balances classical image processing (LoG, morphology, watershed) with careful calibration. Always validate physical units and spot-check overlays. For manuscript figures, prefer fixed random seeds, consistent intensity normalization, and record parameter sets alongside results.